Structural-Functional Divergence During Submarine-Cable Disruptions: Evidence from Yemen's FALCON Dependence

Ezzaldeen Al-Tayar¹, Abdullateef Ali¹,*

¹ Faculty of Engineering and Information Technology, Taiz University, Taiz, Yemen
Ezzaldeen Al-Tayar: azzaltayyar@gmail.com
* Corresponding author: Abdullateef Ali, abullateef.ali@taiz.edu.ye

Author contributions: Ezzaldeen Al-Tayar: Conceptualization, Investigation, Writing – original draft. Abdullateef Ali: Investigation, Data curation, Resources. Both authors reviewed and approved the final manuscript.

Abstract

National Internet resilience can be overstated when control-plane visibility is treated as equivalent to user-facing service. This study examines Yemen’s dependence on the FALCON submarine cable using four analytically distinct measurement families: Border Gateway Protocol (BGP) control-plane visibility, active reachability, reported international capacity, and country-level connectivity/traffic indicators. The January 2020 FALCON cut is the primary event. Public reporting indicates an approximately 80% loss of international capacity for almost a week, while independent monitoring documents loss of an AS15412 dependency and rapid failover by Autonomous System 30873 (AS30873). Bounded digitization of published normalized Internet Outage Detection and Analysis (IODA) plots shows that, during the acute AS30873 window, the BGP-visibility proxy retained approximately 95.7% of its pre-event median whereas the active-probing proxy retained approximately 44.8%, a descriptive divergence of about 50.8 percentage points. A full-factorial sensitivity grid kept the divergence between 45.8 and 55.2 points. A frozen 9 August 2026 BGP-origin snapshot is highly concentrated (Herfindahl–Hirschman Index ≈0.624 for IPv4 prefix shares and ≈0.921 for unique /24-equivalent address space) and is used only as present-day context. The scheduled November 2023 FALCON maintenance event provides a cross-event transferability challenge, indicating that the available cases do not support a universal event-independent recovery parameterization. Route-level historical BGP metrics and native IODA time-series inference remain withheld pending validated primary-data reconstruction.

Keywords: Submarine cables; Internet resilience; BGP; structural-functional divergence; critical infrastructure; Yemen.

1. Introduction

Submarine telecommunications cables form the physical backbone of international digital communications. The International Telecommunication Union (ITU) states that these systems carry more than 99% of global data traffic and identifies physical exposure, repair delay, geographic concentration, and dependence on a small number of cable systems as major resilience challenges [1]. In July 2026, the ITU-ICPC International Advisory Body on Submarine Cable Resilience approved recommendations emphasizing route diversity, infrastructure redundancy, improved risk identification, preparedness, and faster repair [2]. These priorities are especially consequential for underserved or geographically constrained countries whose international connectivity can converge on a small number of physical and logical dependencies.

Yemen provides a technically important event-based empirical case setting because international connectivity has historically depended heavily on the FALCON submarine cable system while backup options have been limited, operationally constrained, or incompletely activated. A cable failure therefore cannot be understood only as a physical infrastructure event. It can cause Border Gateway Protocol (BGP) routing withdrawal or path switching, expose national transit concentration, affect prefixes belonging to multiple organizational operators, degrade service even when some BGP reachability remains, and produce long recovery trajectories that are not captured by static vulnerability metrics.

Recent Internet measurement research has substantially advanced cross-layer cable/IP cartography and physical-network exposure analysis. Nautilus [3] maps likely IP links to candidate submarine cables with confidence scores, Xaminer [4] evaluates cross-layer physical/network exposure, and Calypso [5] maps traceroute-derived digital routes across submarine systems. Recent multilayer studies further show that multilayer representation and shared physical risk are not, by themselves, sufficient novelty claims [6,7]. Multi-source outage research has also established that control-plane visibility can diverge from data-plane or active reachability during large Internet disruptions [8,9]. Accordingly, this study does not claim the first cable-to-IP map, the first multilayer Internet model, the first cascading-failure model, or the first observation of control-/forwarding-plane decoupling. Its narrower empirical question is how a large structural-functional divergence manifests during a physically anchored submarine-cable disruption in a highly dependent national critical-infrastructure setting, and what that evidence implies for redundancy and recovery interpretation.

The study therefore uses the Red Sea-Yemen connectivity setting as an event-based resilience case rather than as a claim of unique geography. A three-layer dependency architecture (submarine infrastructure, international routing, and national autonomous-system (AS)/BGP resources) is retained as an organizing model, but the released quantitative claims are limited to evidence that is actually instantiated and validated.

1.1 Research Questions

RQ1. How did control-plane and functional-service indicators respond during the January 2020 FALCON disruption?

RQ2. To what extent did aggregate routing visibility diverge from active reachability during the acute AS30873 disruption window?

RQ3. What does the current resource-origin concentration pattern reveal about potential common dependencies without assuming that the 2026 snapshot represents the 2020 topology?

RQ4. How transferable are event-specific severity and recovery patterns from the unplanned 2020 FALCON cut to the scheduled 2023 FALCON maintenance event?

1.2 Contributions

C1. A measurement-family-specific resilience design that keeps routing visibility, active reachability, reported capacity, and country-level connectivity/traffic indicators analytically distinct.

C2. An event-specific structural-functional divergence result for AS30873 during the January 2020 FALCON disruption, released only at the aggregate signal-proxy level supported by the available evidence.

C3. A resource-origin dependency analysis that uses the current BGP snapshot as present-day concentration context while explicitly avoiding historical back-projection to 2020.

C4. A cross-event transferability analysis contrasting the unplanned 2020 cut with the scheduled 2023 maintenance event without treating heterogeneous measurement families as directly interchangeable.

C5. A reproducible evidence-provenance and robustness workflow that withholds unsupported route-level, stochastic-recovery, and machine-learning claims, and quantifies the sensitivity of the raster-derived central result rather than treating digitization choices as fixed.

2. Related Work and Research Gap

2.1 Cross-Layer Submarine Cable Mapping

Nautilus [3] established that cross-layer Internet cartography can map IP links to likely submarine cables using public datasets, geolocation, traceroute evidence, and confidence scoring. Calypso [5] extends the evidence base using traceroute and real-world validation to associate digital routes with submarine cables. Weaver et al. [10] address a complementary measurement problem: estimating submarine-segment latency from deployed traceroute infrastructure with path-change-aware methods, while showing that existing measurement systems cannot generally recover dynamic latency variability caused by congestion. Together, these studies motivate confidence-aware mapping and careful separation between what a measurement signal can observe and what it cannot. The present study therefore treats cable/path association as an inferred input rather than a novelty claim.

2.2 Cross-Layer Exposure, Multilayer Robustness, and Cascading Failure

Xaminer [4] is the closest methodological comparator because it evaluates interactions between physical and network-layer failures and produces cross-layer risk profiles. Recent multilayer and shared-risk work [6,7], together with broader telecommunications and interdependent-infrastructure resilience studies [11,12], further narrows any architecture-only novelty claim. Complementary resilience-design literature addresses trans-oceanic cable systems, multilevel redundancy, path diversification, optical survivability, disaster-aware carrier networks, and risk-zone-diversified network design [19-24]. The present paper therefore does not position the three-layer representation itself as the discovery. Instead, the empirical contribution is the event-based comparison of distinct measurement families and the interpretation of their divergence under a known physical cable disruption. The dependency architecture is used to interpret this evidence and to define testable conditions for effective, rather than nominal, redundancy.

2.3 Control-Plane and Forwarding-Plane Divergence in Internet Outages

Multi-source Internet outage research has long shown that control-plane evidence should be interpreted alongside data-plane or reachability measurements. Dainotti et al. [8] combined BGP control-plane observations with several data-plane sources to characterize country-scale outages and showed that the two perspectives need not agree. More recently, Jahromi and Jaskolka [9] analyzed Iranian shutdowns using passive scans, active probing, and RIPE RIS BGP snapshots and found that BGP announcements can remain stable while forwarding-plane reachability collapses. These studies establish that control-/forwarding-plane decoupling is not itself a new phenomenon.

The contribution here is therefore not the generic observation that BGP visibility can persist during functional loss. The narrower novelty is an event-specific demonstration of a large structural-functional divergence during a physically anchored submarine-cable disruption in a highly dependent national critical-infrastructure setting, coupled with explicit measurement-family separation, common-dependency interpretation, and reproducible sensitivity analysis of the available evidence.

2.4 Dynamic Resilience and Uncertainty

Dynamic resilience research motivates treating disruption and recovery as time-dependent [13,16-18], while uncertainty and sensitivity studies emphasize explicit treatment of epistemic uncertainty and parameter dependence [14,15]. The available Yemen event record, however, does not support estimation of a stochastic holding-time model. Accordingly, the main analysis uses an explicit state-transition architecture and event windows. A semi-Markov extension remains a future modeling option if a sufficiently large and measurement-consistent event sample becomes available; it is not presented here as an estimated result or a core contribution.

2.5 Research Gap

The research gap addressed here is narrower than a generic multilayer, cascading-failure, or control-/forwarding-plane divergence gap. Prior work already establishes that control-plane and data-plane evidence can decouple during national outages [8,9], and cross-layer tools can identify likely cable/IP associations and physical-network exposure [3-5]. What remains underdeveloped is event-specific resilience inference for a known submarine-cable failure in a highly dependent national setting when heterogeneous evidence families must be kept separate and common dependencies interpreted cautiously. This paper addresses that measurement and interpretation gap by documenting the 2020 structural-functional divergence, contextualizing present-day resource-origin concentration without historical back-projection, and testing cross-event transferability without forcing heterogeneous evidence into a single scalar performance trace. Table 1 summarizes the closest methodological comparators and the boundary addressed by this study.

Table 1 Closest methodological comparators and the research boundary addressed by the present study

| Work | Established contribution | Overlap with this study | Remaining boundary addressed here |
| --- | --- | --- | --- |
| Nautilus [3] | Confidence-scored IP↔submarine-cable mapping | Cross-layer mapping input | Not an event-based national comparison of control-plane and functional-service signals |
| Xaminer [4] | Cross-layer physical/network exposure and risk profiles | Physical↔network failure analysis | No Yemen event-based comparison of routing visibility and active reachability |
| Calypso [5] | Traceroute-based digital-route↔cable mapping | Mapping/validation evidence | Not a cross-event, measurement-family-specific national resilience analysis |
| Lang et al. [6] | ISP/IXP multilayer model + SRLG robustness | Multilayer/shared physical risk | Static connectivity robustness; not this event-specific multi-signal divergence analysis |
| Wu & Neumueller [7] | Multiplex physical/overlay cascade + historical cable faults | Cascading infrastructure coupling | Different overlay/system objective; no national event-based structural-functional signal comparison |
| Dainotti et al. [8] | Multi-source BGP and data-plane outage characterization | Establishes need for cross-signal outage measurement | Not a submarine-cable-anchored Yemen critical-infrastructure case |
| Jahromi & Jaskolka [9] | BGP + active reachability/passive scans during Iran shutdowns; control/forwarding-plane decoupling | Direct conceptual overlap with structural-functional divergence | Administrative forwarding-plane shutdown mechanism rather than a physically anchored submarine-cable failure; different dependency objective |
| Present study | Event-based measurement-family separation, raster-sensitivity testing, and dependency interpretation | Uses cross-layer architecture as context, not novelty by itself | Physically anchored structural-functional divergence, event-context transferability, and frozen present-day origin-dependence context; route-level multilayer execution withheld |

3. Study Context and Scope

The empirical setting comprises Yemen's international connectivity through the Red Sea and adjacent routes, observable international transit relationships, national AS/prefix resources, and public functional-connectivity evidence. The study intentionally excludes proprietary traffic matrices, operator-internal router topology, precise sensitive infrastructure coordinates, and unverified capacity values. Evidence is classified as OBSERVED, INFERRED, DERIVED, or SIMULATED. The three-layer dependency model is used as an organizing architecture; because route-level historical reconstruction is not yet validated, the paper does not claim a fully instantiated event-time supra-adjacency network or topology-wide cascade effect sizes. Table 2 summarizes the operational definitions and evidence classes used in the organizing architecture.

Table 2 Operational definitions and evidence classes for the dependency architecture

| Element | Operational definition | Evidence class |
| --- | --- | --- |
| Physical submarine layer | Cable systems, landing dependencies, cable–landing relationships | Public/authoritative or confidence-aware inferred |
| International routing layer | Upstream/transit ASes and observed BGP paths connecting Yemen externally | Observed/inferred from RIS, RouteViews, CAIDA |
| National logical layer | Yemeni ASes, originated prefixes, provider/peer dependencies, resource–origin relationships | Observed/inferred |
| Functional measurement families | Q_control(t), Q_active(t), Q_capacity(t), and Q_connectivity(t), normalized within source/family where defensible | Observed or derived; not collapsed into one latent Q(t) |

4. Data Sources, Provenance, and Event Design

Table 3 summarizes the primary data sources, their analytical roles, and their principal limitations.

Table 3 Primary data sources, analytical uses, and key limitations

| Source | Use | Key limitation |
| --- | --- | --- |
| RIPE Routing Information Service (RIPE RIS) / RIPEstat [25] | Historical/current BGP state and updates | Collector visibility; historical update access requires archive/BGPStream |
| Center for Applied Internet Data Analysis (CAIDA) AS Relationships / BGPStream [26] | AS relationship inference and historical MRT processing | Relationships are inferred; not operator ground truth |
| RIPE Atlas [27] | round-trip time (RTT), loss, traceroute, external-to-Yemen measurements | In-country probe coverage may be sparse |
| Internet Outage Detection and Analysis (IODA) [28] | AS/country BGP visibility and active-probing time-series evidence | Signals measure different constructs; current manuscript uses plot digitization plus quantitative sensitivity analysis, while native API samples remain preferred. |
| Cloudflare Radar | Observed traffic/connectivity validation | Not topology ground truth |
| TeleGeography/public cable maps | Cable systems and landing context | Public routes are stylized; detailed data can be licensed |
| World Bank / operator notices / event reports | Historical event severity, recovery, and causal anchors | Different sources measure different quantities |

4.1 Historical Event Hierarchy

Table 4 summarizes the historical events and their roles in the analysis.

Table 4 Historical event hierarchy and role in the analysis

| Event | Role | Observed evidence used |
| --- | --- | --- |
| 9 Jan 2020 FALCON cut | Primary empirical case | ~80% international capacity loss; 20-30% emergency service; AS15412 loss and failover reported [29,30] |
| 30 Jan 2020 second disruption | Contextual second disruption only | Second several-hour national disruption during the continuing FALCON outage; renewed degradation is supported [30], but the exact additional failed physical/backup component remains lower confidence. |
| 21–25 Jan 2022 Yemen outage | Qualitative different-failure-mode challenge | Near-total nation-scale connectivity loss lasting ~4 days [31] |
| 10 Nov 2023 FALCON maintenance | Same-component cross-event transferability challenge | Confirmed scheduled FALCON maintenance; national connectivity reported at 33%; main disruption 00:10-04:45 UTC [32-35] |
| 2024/2025 Red Sea disruptions | Context only | Regional multi-cable and rerouting evidence; used cautiously for Yemen-specific inference |

4.2 Aggregate IODA Structural–Functional Proxy

IODA [28] reports distinct connectivity signals rather than a single interchangeable measure. Its BGP signal aggregates the number of /24 networks visible through full-feed Route Views and RIPE RIS peers, whereas its active-probing signal aggregates /24 networks inferred to be up by active measurement. These signals answer different questions and are therefore normalized and interpreted separately in this study. The 2020 evidence used in the present analysis comes from published normalized IODA plots and is digitized only as a bounded descriptive fallback. The official IODA application programming interface (API) exposes historical raw time-series signals; native samples remain the preferred primary-data upgrade. Because native samples could not be retrieved in the current execution environment, the raster result is accompanied by a quantitative sensitivity analysis rather than promoted to a stronger inferential claim.

5. Cross-Layer Dependency Architecture

For interpretation, the study retains three structural dependency domains: submarine infrastructure, international routing/connectivity, and national AS/BGP resources. Cross-layer relationships indicate how logical connectivity may depend on common physical resources. The architecture is conceptual unless a dependency edge is supported by public evidence or validated mapping; it is not treated as a fully observed topology. The organizing architecture is shown in Fig. 1.

Functional performance is represented by a vector of measurement-specific states Q_m(t), where m indexes the measurement family. The primary families are Q_control(t) for routing/control-plane visibility, Q_active(t) for active reachability, Q_capacity(t) for reported international capacity, and Q_connectivity(t) for country-level connectivity or traffic indicators. Each family may be normalized to its own pre-event baseline when the source supports that operation. Values from different families are not averaged into a latent Q(t), and resilience integrals are not compared across families unless the measurement semantics and time bases are commensurable.

Fig. 1 Dependency architecture used to interpret physical, routing, and national-AS relationships; functional evidence is analyzed through measurement-specific Q_m(t) signals rather than as a fourth network layer

5.1 Cross-Layer Mapping

Cable/path associations and dependency edges are retained only when supported by public operator information, traceroute geography, BGP-path evidence, landing context, or historical event response. Confidence labels remain useful for future event-time graph reconstruction, but this revision does not report quantitative mapping-branch robustness because the required route-level historical input is not yet validated.

5.2 Resource–Origin Dependency

Resource-origin analysis captures the fact that organizationally distinct resources can share an origin ASN. Representative current observations show this pattern for some Yemen Mobile, YOU, and Sabafon resources associated with AS30873. This is interpreted as evidence that organizational labels alone do not establish routing independence; it is not used to infer that all operator traffic shares one path or that the same concentration held during the 2020 event.

6. Measurement-Specific Resilience and Dependency Analysis

The temporal analysis distinguishes Normal, Disruption, Adaptation, and Recovery phases at the event level. The purpose is descriptive and falsifiable: event windows organize what is observed without assuming memoryless or fitted stochastic transition times. A semi-Markov formulation was pre-specified during project development, but no holding-time distribution can be estimated credibly from the sparse, heterogeneous historical events. It is therefore removed from the main contribution set and retained only as a possible future extension.

6.1 Route-Level Metrics: Definitions and Withholding Rules

Structural quantities such as prefix visibility retention, route-observation retention, upstream switching, provider-set churn, AS-path inflation, cascade size, and cascade depth require valid historical route-state data from comparable vantage points. Those quantities are defined in Online Resource 1 but are not released numerically in this manuscript because the attempted checkpoint bundle failed validation. The main paper therefore avoids claims about which dependency 'most strongly' amplifies failure at the route level.

6.2 Measurement-Family-Specific Functional Resilience

For each measurement family m, normalized retention can be written q_m(t)=X_m(t)/B_m, where B_m is a pre-event baseline defined within that same source and signal family. Maximum loss, area loss, and recovery thresholds are meaningful only within a sufficiently sampled and semantically consistent time series. Because the present evidence does not provide such a continuous, homogeneous trace across the full 2020 recovery period, the manuscript does not release a 27-day scalar resilience-loss integral or T_R(90/95/99) as empirical estimates.

6.3 Structural–Functional Dual Axis

The primary structural-functional comparison is contemporaneous but signal-specific: routing/control-plane retention and active-reachability retention are normalized to their own pre-event medians and compared as separate proportions. Their difference is interpreted as a descriptive divergence between measurement families, not as a difference between two estimates of the same latent quantity.

6.4 AI-Assisted Code Development and Human Verification

ChatGPT (OpenAI) was used to assist with drafting and reviewing selected analysis and reproducibility scripts. It did not generate empirical observations or substitute for source data. All scripts used for reported results were manually inspected, executed against the stated inputs, and their numerical outputs were checked by the authors. Fail-closed validation rules were retained so that empty or unvalidated raw inputs could not be converted into reported results.

7. Event Comparison and Evidence Boundaries

7.1 Cross-Event Transferability Design

The 2020 FALCON cut is the primary empirical case. The 2023 scheduled FALCON maintenance event is used as a same-component cross-event transferability challenge, not as a homogeneous predictive-validation sample, because the operational context and measurement semantics differ. The 2022 national outage is retained as a qualitative different-failure-mode challenge. No parameter is silently refitted to force agreement across events.

7.2 Machine Learning Exclusion

Machine learning is not part of the final contribution set. The available historical events are too few and heterogeneous to support a defensible predictive model, and deterministic scenario-grid rows are not treated as independent empirical samples.

8. Results

8.1 Present-Day BGP-Origin Concentration as Context

A frozen prefix-level public-routing snapshot assembled on 9 August 2026 [36] contains 150 IPv4 prefixes across AS30873, AS12486, and AS204317. AS30873 accounts for 76.0% of the frozen prefix set, AS12486 for 21.33%, and AS204317 for 2.67%, corresponding to an origin-prefix Herfindahl–Hirschman Index (HHI) of approximately 0.624. After collapsing overlapping more-specific prefixes, the unique /24-equivalent address-space shares are approximately 95.88%, 3.67%, and 0.44%, yielding an HHI of approximately 0.921. These values establish present-day origin concentration in the assembled snapshot only. They are not interpreted as the routing state of January 2020, market share, subscriber share, or traffic share. Historical concentration reconstruction remains a separate empirical task. The corresponding concentration shares are summarized in Table 5 and visualized in Fig. 2.

Fig. 2 Present-day BGP-origin concentration in the frozen assembled 9 August 2026 snapshot; the result is contextual and is not back-projected to the 2020 disruption

Table 5 Present-day BGP-origin concentration in the frozen 9 August 2026 snapshot

| Origin ASN | Share of observed IPv4 prefixes | Share of unique /24-equivalent originated IPv4 space |
| --- | --- | --- |
| AS30873 | 76.00% | 95.88% |
| AS12486 | 21.33% | 3.67% |
| AS204317 | 2.67% | 0.44% |

8.2 2020 FALCON Disruption: Structural Adaptation with Severe Functional Loss

The January 2020 FALCON cut is the primary event. World Bank reporting indicates an approximately 80% loss of Yemen's international Internet capacity for almost a week [29]. Independent event monitoring reports a much stronger impact on AS30873 than AS12486, loss of an AS15412 dependency, and rapid AS30873 failover to alternate upstream providers [30]. This combination is important because it documents routing adaptation while functional service remained severely constrained; however, it does not by itself quantify route-level retention or path diversity.

Historical route-level reconstruction remains unresolved in this revision. The attempted RIPEstat checkpoint bundle returned empty route states and was rejected. Official RIPE RIS MRT archives [25] and CAIDA BGPStream [26] provide a technically appropriate route to reconstruct historical RIB/update evidence from multiple collectors, and scripts for that primary-data upgrade are supplied in Online Resource 1. Until that execution passes a comparable-vantage-point validity check, prefix-visibility retention, route-observation retention, upstream switching, provider churn, AS-path inflation, and route-level path diversity remain withheld.

8.2.1 Aggregate IODA Evidence of Structural-Functional Divergence

Using the published normalized IODA plots as a bounded fallback, AS30873 retained approximately 95.7% of its pre-event median BGP-visibility proxy during the acute 18:00-00:00 UTC window, whereas the active-probing proxy retained approximately 44.8%. The descriptive difference is therefore about 50.8 percentage points. AS12486 retained approximately baseline-level median values in both plotted signals across the same windows, although its active-probing trace shows a brief dip. Because IODA's BGP and active-probing signals are different measurement families, this 50.8-point difference is interpreted as structural-functional signal divergence, not as a route-level loss fraction or as two measurements of an identical latent service variable. The signal-family comparison is shown in Fig. 3.

Raster digitization is an acknowledged measurement limitation. To test whether the central result depends on reasonable extraction choices, a pre-specified full-factorial sensitivity grid perturbed the pre-event baseline start (±2 h), baseline end (±60 min), acute-window start and end (each ±60 min), y-axis zero (±2 pixels), y-axis scale (±2 pixels per normalized unit), red-green-blue (RGB) matching tolerance (0-2 per channel), and the summary statistic (median versus mean). The grid generated 20,250 valid parameter settings, which should not be interpreted as independent experiments. Across the grid, the AS30873 BGP-minus-active retention divergence ranged from 45.8 to 55.2 percentage points; the 5th-95th percentile range was 47.5-53.4 points around the central estimate of 50.8 points. Y-axis scale and RGB-tolerance perturbations were invariant within numerical precision over the tested ranges. This robustness analysis does not convert raster-derived values into native IODA samples, but it shows that the qualitative structural-functional divergence is not an artifact of one fixed digitization/window choice.

Fig. 3 Aggregate IODA signal retention during the 9 January 2020 acute window; BGP visibility and active probing are normalized within their own signal families, and the AS30873 central divergence of 50.8 percentage points remains 45.8–55.2 points across the raster-sensitivity grid

8.3 January 2022 Outage as a Qualitative Failure-Mode Challenge

The January 2022 event is retained as an external qualitative failure-mode challenge. Public reporting indicates a near-total nationwide connectivity disruption lasting about four days [31], but the accessible evidence does not establish a sufficiently complete and homogeneous physical-to-routing causal chain for quantitative scoring in the same manner as the 2020 FALCON event. Accordingly, the event is not presented as an equivalent numerical holdout.

8.4 November 2023 FALCON Maintenance as a Transferability Challenge

The scheduled FALCON maintenance event of 10 November 2023 offers a same-component but operationally different comparison. Cloudflare reported a country- and AS30873-level disruption from approximately 00:10 to 04:45 UTC [32], NetBlocks reported national connectivity at 33% of ordinary levels during the severe period [33], and Global Cloud Xchange confirmed scheduled maintenance [34]; contemporary reporting also documented the outage context [35]. The event is therefore used to test whether qualitative severity/recovery interpretations developed from the unplanned 2020 cut transfer to a planned maintenance regime. Because the underlying signals are not identical measurement families, no predictive error such as |0.20-0.33| is reported. Table 6 summarizes the cross-event comparison and interpretive role.

Table 6 Cross-event comparison and interpretive role

| Event | Functional severity | Duration/recovery anchor | Interpretive role |
| --- | --- | --- | --- |
| FALCON 2020 | ~80% capacity loss; emergency service 20–30% | Severe loss reported for almost a week; physical repair later reported separately | Primary empirical case; capacity evidence not treated as a universal latent Q(t) |
| Yemen outage 2022 | Near-total national connectivity loss | ~4 days | Qualitative failure-mode challenge |
| FALCON maintenance 2023 | National connectivity reported at 33% at severe point | ~275 min main disruption | Same-component transferability challenge |

8.5 Main Evidence-Constrained Findings

Finding 1: The 2020 FALCON event documents routing adaptation in AS30873 while national functional capacity remained severely constrained, showing that adaptation is not equivalent to service restoration.

Finding 2: In the published normalized IODA evidence for the acute AS30873 window, the BGP-visibility proxy retained about 95.7% of its pre-event median while the active-probing proxy retained about 44.8%, a descriptive cross-signal divergence of about 50.8 percentage points; this divergence remained 45.8-55.2 points across the raster-sensitivity grid.

Finding 3: The 9 August 2026 BGP snapshot is highly concentrated at the origin level, but this is treated as present-day context rather than evidence that the same concentration existed in January 2020.

Finding 4: The 2023 scheduled FALCON maintenance event is materially different from the unplanned 2020 cut in operational context and duration, so a universal event-independent recovery parameterization is not supported.

Finding 5: Route-level BGP change metrics, native-IODA inferential statistics, scalar 27-day resilience-loss estimates, and machine-learning/explainable-AI performance remain withheld pending valid primary-data execution.

8.6 Sensitivity Analysis of Plot-Derived IODA Evidence

The raster-sensitivity analysis evaluates the stability of the primary descriptive result under reasonable alternatives for event-window placement, baseline placement, axis calibration, line-color matching, and mean-versus-median aggregation. Across the full-factorial grid, the minimum observed AS30873 divergence was 45.8 percentage points and the maximum was 55.2 points, with 90% of evaluated settings between 47.5 and 53.4 points. The grid generated 20,250 valid parameter settings, but these are not independent experiments; y-axis scale and RGB-tolerance dimensions were invariant within numerical precision over the tested ranges. The direction and practical magnitude of the structural-functional gap therefore persist across the tested extraction choices, while the evidence class remains DERIVED_FROM_PUBLISHED_NORMALIZED_PLOT.

9. Robustness and Evidence Boundaries

The principal robustness rule is measurement and provenance consistency. Observed, inferred, derived, and simulated quantities are not promoted across evidence classes. For the central plot-derived IODA result, robustness is additionally tested through a full-factorial digitization/window sensitivity grid; the structural-functional divergence remains 45.8-55.2 percentage points. The generated settings are not treated as independent experiments, and dimensions that were invariant over the tested range are reported as such. The 2020 and 2023 events are compared only for transferability of interpretation, and heterogeneous signals are not converted into a predictive-error statistic. The 2022 event remains a qualitative failure-mode challenge. Unvalidated route-level metrics and mapping-branch effect sizes remain withheld until historical MRT/BGPStream reconstruction and event-time cross-layer mapping are executed with comparable vantage points.

The current evidence supports the structural-functional divergence claim at the aggregate measurement-signal level, supports the robustness of that conclusion to reasonable raster extraction choices, and supports present-day origin concentration as contextual evidence. It does not support topology-wide ranking of the strongest dependencies, empirical intervention effect sizes, stochastic semi-Markov estimates, or machine-learning performance.

10. Discussion

10.1 Beyond Provider Counts

The 2020 case supports a distinction between nominal and effective redundancy. AS30873 reportedly failed over after loss of an AS15412 dependency, but service remained severely constrained. This does not prove that a specific alternative physical topology would have solved the event; rather, it demonstrates why counting providers or routes is insufficient without testing whether the alternatives remain independent under the initiating failure mode.

10.2 Why Structural Reachability Is Not Enough

The strongest empirical message of the study is not that control-plane and forwarding-plane evidence can diverge in general; that phenomenon is established in prior outage studies [8,9]. The contribution here is the event-specific interpretation of a large divergence during a physically anchored submarine-cable disruption. In the available IODA plot evidence [28], AS30873's aggregate BGP-visibility proxy remained near its pre-event level while active reachability fell much further. Because IODA defines these as distinct signals, the result is not treated as a discrepancy between two measurements of the same quantity. Raw IODA exports and historical MRT reconstruction would strengthen the event analysis by removing the plot-digitization fallback and enabling route-level inference.

10.3 Recovery as a Distinct Dimension

Recovery remains a distinct dimension, but the present evidence does not justify a single continuous national Q(t) trace across the full 2020 recovery period. Capacity reports, active probing, BGP visibility, and 2023 connectivity measurements are therefore kept separate. The contrast between the prolonged unplanned 2020 disruption and the much shorter scheduled 2023 maintenance event illustrates that an event-independent recovery parameterization is not supported by the available cases. Because the events differ in operational context and measurement family, this comparison is not presented as a general law of event-class dependence or as a direct comparison of commensurable resilience integrals.

10.4 Policy and Engineering Implications

For resilience planning, the defensible implication is to test independence under the relevant failure mode rather than rely on nominal provider counts. A logically different upstream that still shares the failed physical dependency cannot protect against that same failure mode by definition; benefit from physical diversity should therefore be treated as an engineering design condition to be verified in an instantiated topology, not as an empirical intervention effect demonstrated by this paper. This interpretation is consistent with international emphasis on geographic diversity, infrastructure redundancy, preparedness, and timely repair [1,2] and with broader network-resilience design principles [20,24]. The paper does not estimate the cost, deployable capacity, or optimal siting of alternative infrastructure, and it intentionally excludes sensitive operational location details.

10.5 Relation to Contemporary Cross-Layer and Multilayer Work

Nautilus [3] and Calypso [5] inform the mapping problem, Xaminer [4] provides a benchmark for cross-layer exposure analysis, and recent multilayer/shared-risk work [6,7] demonstrates that architecture alone is no longer a sufficient novelty claim. Likewise, earlier and contemporary outage studies already establish control-/forwarding-plane divergence as a general measurement phenomenon [8,9]. The contribution retained here is therefore deliberately narrower: an event-based, measurement-aware demonstration of a large divergence during a known submarine-cable disruption, a quantitative robustness analysis of the available raster-derived signal comparison, and disciplined interpretation of present-day resource-origin concentration. A future raw-data extension using native IODA time series and BGPStream/MRT would permit stronger event-time multilayer and route-level claims that are intentionally withheld in this version.

11. Limitations

Public routing and measurement data provide partial Internet visibility and cannot recover proprietary operator traffic matrices or internal topology.

Public submarine cable maps often generalize cable routes; precise route coordinates are intentionally neither required nor reported.

Historical evidence sources measure different constructs (capacity, BGP visibility, active reachability, and country-level connectivity/traffic). They are analyzed as separate measurement families and are not treated as interchangeable observations of one latent national-performance variable.

The 2020 IODA structural-functional comparison is digitized from published normalized raster plots and is suitable only for bounded descriptive comparison. A full-factorial sensitivity grid shows that the reported divergence remains 45.8-55.2 percentage points under reasonable digitization and window perturbations. The grid generated 20,250 valid settings, not 20,250 independent experiments, and y-axis scale and RGB-tolerance perturbations were invariant within numerical precision over the tested ranges. This robustness check does not remove the source limitation. Native IODA historical samples remain preferable for timestamp-level inference, recovery trajectories, and formal uncertainty intervals [28].

Route-level 2020/2022/2023 BGP metrics are not released. One RIPEstat checkpoint retrieval returned empty route states and failed validation; this does not establish that historical reconstruction is impossible. RIPE RIS MRT archives [25] and CAIDA BGPStream [26] remain the required primary-data route for future PVR, ROR, provider-churn, AS-path, and path-diversity analysis.

Representative mobile-operator prefixes sharing AS30873 demonstrate a resource–origin dependency pattern but do not prove that every resource or all operator traffic shares the same path.

The study does not estimate empirical intervention effects or the available capacity of a physically independent alternative. The discussion of shared versus independent failure dependencies is an engineering implication of the dependency definitions and should not be read as measured superiority of a deployed intervention.

Machine learning and explainable artificial intelligence are omitted because the available historical events are too few and heterogeneous, and deterministic scenario-grid rows do not constitute independent empirical training evidence.

The Red Sea–Yemen corridor is an analytical scope definition and should not be interpreted as a formally designated telecommunications corridor.

12. Reproducibility and Research Integrity

The study separates OBSERVED, INFERRED, DERIVED, and SIMULATED evidence and uses a pre-specified, versioned analysis workflow whose rules were frozen before the corresponding execution steps. The term 'pre-specified' is not used to imply an external timestamped registry. Online Resource 1 is organized around a reviewer-facing README, data, code, results, figures, and provenance. A single reproduce_all.py entry point recalculates the present-day HHI values directly from the frozen prefix-level snapshot, the plot-derived IODA retention/divergence, raster-sensitivity results, and Figures 2-3 from locally included evidence. Primary-data acquisition scripts for IODA and BGPStream are included separately and fail explicitly when network access or valid raw inputs are unavailable.

13. Conclusion

This study shows why national Internet resilience should not be inferred from control-plane visibility or provider counts alone. During Yemen's January 2020 FALCON disruption, independent reporting documented severe functional-capacity loss despite routing adaptation [29,30]. In the available normalized IODA evidence, AS30873 retained approximately 95.7% of its pre-event BGP-visibility proxy while the active-probing proxy retained approximately 44.8% during the acute window, producing an event-specific descriptive divergence of about 50.8 percentage points. Across the full-factorial raster/window sensitivity grid, the divergence remained 45.8-55.2 percentage points, strengthening the conclusion that the observed gap is not driven by one digitization choice while leaving the evidence explicitly plot-derived. A frozen 2026 routing snapshot further shows high origin concentration, but it is not back-projected to the 2020 event. The 2023 scheduled FALCON maintenance event provides a same-component transferability challenge and indicates that the available cases do not support a universal event-independent recovery parameterization. The paper therefore retains a narrow but defensible conclusion: effective resilience assessment requires measurement-specific evidence and independence that survives the relevant physical failure mode. Native IODA time-series extraction and historical MRT/BGPStream reconstruction remain valuable empirical upgrades before topology-wide cascade, intervention-effect, or route-level claims can be released.

References

[1] International Telecommunication Union. (2026). International Advisory Body approves landmark report to strengthen submarine cable resilience. ITU, 10 July 2026.

[2] International Telecommunication Union & International Cable Protection Committee. (2026). IAB Working Group Recommendations: Timely Deployment & Repair; Risk Identification, Monitoring & Mitigation; Fostering Connectivity & Geographic Diversity. Accessed 9 August 2026.

[3] Ramanathan, A., & Abdu Jyothi, S. (2023). Nautilus: A framework for cross-layer cartography of submarine cables and IP links. Proceedings of the ACM on Measurement and Analysis of Computing Systems, 7(3), Article 46. https://doi.org/10.1145/3626777

[4] Ramanathan, A., Sankaran, R., & Abdu Jyothi, S. (2024). Xaminer: An Internet cross-layer resilience analysis tool. Proceedings of the ACM on Measurement and Analysis of Computing Systems, 8(1), Article 16. https://doi.org/10.1145/3639042

[5] Wang, C. J., Zhang, Y., Dong, Q., Carisimo, E., Durairajan, R., & Bustamante, F. E. (2025). Threading the ocean: Mapping digital routes across submarine cables using Calypso. ACM SIGCOMM 2025. https://doi.org/10.1145/3718958.3750512

[6] Lang, B., Roughan, M., & Ye, M. (2026). Robustness analysis of Australia's Internet using a multilayer network model. arXiv:2606.18737. https://doi.org/10.48550/arXiv.2606.18737

[7] Wu, W., & Neumueller, A. (2026). Bitcoin under stress: Measuring infrastructure resilience 2014–2025. arXiv:2602.14372. https://doi.org/10.48550/arXiv.2602.14372

[8] Dainotti, A., Squarcella, C., Aben, E., Claffy, K. C., Chiesa, M., Russo, M., & Pescapè, A. (2014). Analysis of country-wide Internet outages caused by censorship. IEEE/ACM Transactions on Networking, 22(6), 1964–1977. https://doi.org/10.1109/TNET.2013.2291244

[9] Jahromi, A. S., & Jaskolka, J. (2026). A multi-perspective study of the Internet shutdown in Iran. arXiv:2605.00187. https://doi.org/10.48550/arXiv.2605.00187

[10] Weaver, M., Veitch, D., Barford, P., Bustamante, F. E., & Carisimo, E. (2026). Monitoring latency on submarine cables: Limitations and opportunities. 1st New Ideas in Networked Systems (NINeS 2026), OASIcs, 139, 16:1–16:24. https://doi.org/10.4230/OASIcs.NINeS.2026.16

[11] Owen, R., Bryant, A. R., Finch, L., Franklin, D., Abdollahi, M., & Abolhasan, M. (2025). Failures and resilience in the IP era: Navigating the fragility of modern telecommunications networks—The sovereign functions. IEEE Access, 13, 155759–155777. https://doi.org/10.1109/ACCESS.2025.3602054

[12] Venkatasubramanian, B. V., Laoudias, C., & Panteli, M. (2025). Cascading failures and resilience in interdependent critical infrastructures: A dynamic approach. IEEE Systems Journal. https://doi.org/10.1109/JSYST.2025.3629327

[13] Caetano, H. O., Desuó Neto, L., Fogliatto, M. S. S., & Maciel, C. D. (2024). Resilience assessment of critical infrastructures using dynamic Bayesian networks and evidence propagation. Reliability Engineering & System Safety, 241, 109691. https://doi.org/10.1016/j.ress.2023.109691

[14] Cavalieri, F., & Franchin, P. (2020). Seismic risk of infrastructure systems with treatment of and sensitivity to epistemic uncertainty. Infrastructures, 5(11), 103. https://doi.org/10.3390/infrastructures5110103

[15] Liu, X., Zio, E., Borgonovo, E., & Plischke, E. (2024). A systematic approach of global sensitivity analysis and its application to a model for the quantification of resilience of interconnected critical infrastructures. Energies, 17(8), 1823. https://doi.org/10.3390/en17081823

[16] Ferrario, E., Pedroni, N., & Zio, E. (2015). Analysis of the robustness and recovery of critical infrastructures within a multistate system-of-systems framework in the presence of epistemic uncertainty. ASCE-ASME Journal of Risk and Uncertainty in Engineering Systems, Part B, 1(3), 031001. https://doi.org/10.1115/1.4030439

[17] Johansen, C., & Tien, I. (2018). Probabilistic multi-scale modeling of interdependencies between critical infrastructure systems for resilience. Sustainable and Resilient Infrastructure, 3(1), 1–15. https://doi.org/10.1080/23789689.2017.1345253

[18] Mottahedi, A., Sereshki, F., Ataei, M., Nouri Qarahasanlou, A., & Barabadi, A. (2021). The resilience of critical infrastructure systems: A systematic literature review. Energies, 14(6), 1571. https://doi.org/10.3390/en14061571

[19] Omer, M., Nilchiani, R., & Mostashari, A. (2009). Measuring the resilience of the trans-oceanic telecommunication cable system. IEEE Systems Journal, 3(3), 295–303. https://doi.org/10.1109/JSYST.2009.2022570

[20] Sterbenz, J. P. G., Hutchison, D., Çetinkaya, E. K., Jabbar, A., Rohrer, J. P., Schöller, M., & Smith, P. (2014). Redundancy, diversity, and connectivity to achieve multilevel network resilience, survivability, and disruption tolerance. Telecommunication Systems, 56(1), 17–31. https://doi.org/10.1007/s11235-013-9816-9

[21] Rohrer, J. P., Jabbar, A., & Sterbenz, J. P. G. (2009). Path diversification: A multipath resilience mechanism. In 7th International Workshop on Design of Reliable Communication Networks (DRCN 2009) (pp. 343–351). https://doi.org/10.1109/DRCN.2009.5339988

[22] Ashraf, M., Idrus, S. M., Iqbal, F., Butt, R. A., & Faheem, M. (2018). Disaster-resilient optical network survivability: A comprehensive survey. Photonics, 5(4), 35. https://doi.org/10.3390/photonics5040035

[23] de Sousa, A., Rak, J., Barbosa, F., Santos, D., & Mehta, D. (2020). Improving the survivability of carrier networks to large-scale disasters. In J. Rak & D. Hutchison (Eds.), Guide to Disaster-Resilient Communication Networks (pp. 175–192). Springer. https://doi.org/10.1007/978-3-030-44685-7_7

[24] Wan, Y., Feng, C., Wu, K., & Wang, J. (2024). RZDD: Risk zone-diversified network design for disaster resilience. IEEE Transactions on Dependable and Secure Computing. https://doi.org/10.1109/TDSC.2024.3406703

[25] RIPE NCC. (2024). Routing Information Service (RIS): Route Collection Raw Data—MRT Files documentation.

[26] CAIDA. (2026). BGPStream documentation and data-provider interfaces for Route Views and RIPE RIS historical BGP data. Accessed 9 August 2026.

[27] RIPE NCC. (2026). RIPE Atlas measurement platform documentation. Accessed 9 August 2026.

[28] Internet Outage Detection and Analysis (IODA). (2026). IODA Resources and HTTP API documentation. Georgia Institute of Technology Internet Intelligence Lab. Accessed 9 August 2026.

[29] World Bank. (2020). Yemen Monthly Economic Update, January 2020. World Bank.

[30] Belson, D. (2020). Internet Disruption Report: January 2020. Internet Disruption Report.

[31] NetBlocks. (2022). Internet connectivity knocked out in Yemen after airstrike. January 2022.

[32] Cloudflare. (2024). Q4 2023 Internet Disruption Summary. Cloudflare Radar/Blog.

[33] NetBlocks. (2023). YemenNet outage metrics: Overall national connectivity at 33% of ordinary levels. 10 November 2023.

[34] Global Cloud Xchange. (2023). Clarification: Yemen Internet Outage. GCX.

[35] Gambrell, J. (2023). Internet collapses in war-torn Yemen over 'maintenance' involving undersea line. Associated Press, 10 November 2023.

[36] bgp.tools. (2026). Public BGP origin snapshots for AS30873, AS12486, and AS204317, frozen/assembled 9 August 2026. Accessed 9 August 2026.

Statements and Declarations

Funding

No funding was received for conducting this study or for the preparation of this manuscript.

Competing interests

The authors have no relevant financial or non-financial interests to disclose.

Data availability

Public or research-accessible evidence sources include RIPE RIS [25], CAIDA BGPStream [26], RIPE Atlas [27], IODA [28], public event reports [29–35], and the public BGP-origin source used for the frozen present-day snapshot [36]. Online Resource 1 contains the plot-derived IODA evidence and provenance used in the manuscript, the complete raster-sensitivity output, a frozen prefix-level BGP-origin snapshot (bgp_origin_snapshot_2026-08-09.csv) from which HHI is recalculated, and primary-data acquisition/reconstruction scripts. Third-party raw data remain subject to provider terms.

Code availability

Online Resource 1 contains a reproduction entry point that recalculates the locally supportable numerical results and figures, together with the IODA raw-signal acquisition script, the historical BGPStream/MRT reconstruction script, and validation checks that prevent empty or non-comparable raw inputs from being converted into results.

Ethics approval

Not applicable. The study used public and aggregate telecommunications and Internet-measurement data and involved no human participants or animals.

Ethics, security and dual-use statement

The study addresses defensive telecommunications resilience. It uses public and aggregate information and intentionally excludes precise vulnerable infrastructure coordinates, operational sabotage instructions, target-selection logic, and unsupported conflict attribution. Historical event attribution is separated from observed disruption evidence unless independently confirmed.

Supplementary information

Online Resource 1 (ESM.zip): reproducibility package containing supporting data, analysis code, robustness results, figures, provenance files, validation logs, and primary-data acquisition/reconstruction scripts.